Circuit Analysis

Laboratory Exercise No: 3

Wheatstone bridge Analysis and Differential Amplifier

Introduction

The main objective of this laboratory exercise is to build, test and characterize a Wheatstone bridge. The Wheatstone bridge can be used to measure resistance, capacitance or inductance of an electronic device when it is in balance. When the Wheatstone bridge is slightly unbalanced due to a small change in one electric device (such as a sensor), the voltage across the bridge is not zero any more. The differential amplifier can be used to amplify this small signal and make it into use. In this lab, you will be introduced to different Wheatstone bridge configurations along with the theory and principle in part I. In part II, you will measure the change of resistance in a quarter-bridge.

Part I

1. Wheatstone Bridge

A Wheatstone bridge is used to convert the change in resistance of a sensor into a voltage output. As shown in Figure 1, the Wheatstone bridge is essentially a circuit consisting of four resistors: 𝑅1, 𝑅2, 𝑅3 and 𝑅4. A voltage 𝑉in is input signal and a voltmeter is used to measure 𝑉out (the potential difference between point A and B).

Figure 1: Typical Wheatstone Bridge

For example, if one of the resistors is used as a sensing element to measure mechanical stress applied to a structure such as aircraft or wind turbine, the stress will change the resistance of the sensing element in the Wheatstone bridge and the voltage (𝑉out) between point A and B changes as a result. The Wheatstone bridge circuit is typically used to measure very small variations in resistance that the sensing element produces in response to stress, temperature and other phenomena. There are three types of Wheatstone bridge configurations: full-bridge, half-bridge and quarter-bridge. The number of active resistors in the Wheatstone bridge determines the type of bridge configuration. Table 1 shows how many active resistors there are in each configuration:

Table 1: Wheatstone bridge configuration

| Configuration | Number of active resistors | Bridge type |
| --- | --- | --- |
| Full-bridge | 4 |  |
| Half-bridge | 2 |  |
| Quarter-bridge | 1 |  |

2. Pre-Lab Questions

a) Analyze the Wheatstone bridge shown in Figure 1 and show that the voltage between point A and B can take the following form:

(1)

b) Determine the relationship among the resistors when the Wheatstone bridge shown in Figure 1 is balanced, i.e. Vout = 0, while

c) When there is a change (∆R) in one of the four resistors, the quarter-bridge becomes unbalanced. Assuming R1 = R2 = R3 = R and R4 = R + ∆R in Figure 1, show that the voltage between point A and B can take the following form

(2)

3. Lab Section 1

In this section, you will learn how to construct a Wheatstone bridge circuit on a bread board and measure the current through each resistor. The Wheatstone bridge circuit to be built is shown in Figure 2. The procedure includes the following steps:

(i) Read the Wheatstone bridge circuit shown in Figure 2,

(ii) Check the resistors needed for this exercise.

Identify the resistors using the method you have learned in Lab 1. Use Multimeter to measure the resistance of each resistor and arrange the resistors in the bridge as balanced as possible, labelling them as R1, R2, R3 and R4.

(iii) Place and connect resistors in the correct configuration on bread board as shown in Figure2.

(iv) Connect the Power Supply to the Wheatstone bridge circuit supplying a voltage of 5V between point C and D.

(v) Using the Digital Multimeter (Ammeter), measure the current flowing through each resistor (i.e. I1, I2, I3, I4 and I5). Mind you should connect the Ammeter in series with the branch you want to measure.

Figure 2: Wheatstone bridge to be constructed

4. Calculations and Discussion Questions for Section 1

a) Using any circuit analysis method, compute the current flowing through each resistor for the Wheatstone bridge and complete Table 2.

b) Compare the currents measured practically with the currents by theoretical calculation.

c) Calculate a percent error of current for each resistor.

d) Based on the measured currents, state whether the circuit is balanced or not.

Table 2: Data Table

| Resistor | Measured resistance | Current | Measured current (mA) | Calculated current (mA) | Error (%) |
| --- | --- | --- | --- | --- | --- |
| R1 |  | I1 |  |  |  |
| R2 |  | I2 |  |  |  |
| R3 |  | I3 |  |  |  |
| R4 |  | I4 |  |  |  |
| R5 |  | I5 |  |  |  |

Part II

1. Differential Amplifier

While using Wheatstone bridge to measure a small change in resistance, the voltage output over the Wheatstone bridge is often typically small. A differential amplifier can be used to amplify this small voltage. As shown in Figure 3, a typical differential amplifier circuit is composed of an operational amplifier and resistors. In this exercise, we use LM324 Op-Amp which includes four op amps. You can select any one from the four op amps.

Figure 3 Typical Differential Amplifier

2. Pre-Lab Questions

a) Analyze the differential amplifier shown in Figure 3 and determine that if R7/R6 = R9/R8, the output voltage Vout can be written as

(3)

Where G is the voltage gain of the Op-Amp, given by:

(4)

b) Using equations (2)(4), determine the resistance change R shown in Figure 4.

3. Lab Section 2

In this section, you will learn how to measure the resistance change by using the output voltage over the bridge you just build in section1. In order to measure a very small change of resistance which means the output voltage over the bridge is very small, you will learn how to build a differential amplifier and use the subsequent output voltage from the amplifier to calculate the resistance change. A good example of small resistance change is strain gauges which are commonly used to measure mechanical strain and vibration in structures such as automobiles, aircrafts, wind turbines and assembled systems. Strain gauges are also used to measure pressure and flow properties and, monitor the environmental conditions in chemical and processing plants such as petrochemical, bioenergy and paper manufacturing plants, to name a few. To make the experiment easier, in this section you will measure an unknown resistance instead of a strain gauge.  The test circuit to be built is shown in Figure 4. The procedure includes the following steps:

Figure 4 Resistance sensing instrumentation using Wheatstone bridge and differential amplifier

(i) Study the resistant sensing instrumentation circuit shown in Figure 4.

(ii) Check the resistors needed and LM324 op amp for this exercise.

(iii) Identify the resistors R6, R7, R8 and R9: R6=R8=47 k, R7 =R9=220 k. Use the Digital Multimeter to measure the resistance of each resistor and then calculate G. Mind that, when you have measured these resistors, compare their resistance and make R7/R6 = R9/R8 as close as possible so that you can get better G.

(iv) Identify the pin connections of LM324. The LM324 Op-Amp consists of four independent high-gain frequency-compensated operational amplifiers that are designed specifically to operate from a single supply or double supplies over a wide range of voltages. The top view of pin connections is shown in Figure 5. In this exercise we only use one op amp. You can select any one from the four op amps.

(v) Based on the Wheatstone bridge you just build, connect the op-amp and resistors in the right configuration on bread board according to Figure 4.

(vi) Connect the Power Supply to the circuit to give 5V (using CH3 or CH4) and ±12V (using CH1 in series with CH2) with a current limit of 300mA accordingly. Do mind not to confuse the + and – connections. Otherwise, the op amp will be ruined. Make sure to connect all negative connectors of the three voltage sources to one reference node (Ground).

Figure 5. LM324 Pin Connections (Top View)

(xii) Use the Digital Multimeter to measure the output voltage from the Wheatstone bridge (across the two test points shown in Figure 4) to verify the operation of the circuit.

(xiii) Use the Digital Multimeter to measure the output voltage from LM324 to verify equation (3).

4. Calculations and Discussion Questions

a. Calculate the theoretical gain G of the Op-Amp according to Equation (4).

b. Calculate the fractional change of the electrical resistance. Recall that Vout = Vin * (∆R/4R), Vin is the voltage applied to the Wheatstone bridge (i.e., 5V) and Vout is the potential difference at the output of the Wheatstone bridge.

c. Complete Table 3.

f. Compare the gains measured practically with the computed theoretical gain.

g. Calculate the percent error.

h. Comment on any variation of the results.

Table 3: Data Table for measurement

| Output before amplification Vout(mV) | Output after amplification Vout(mV) | R/R | Measured Gain | Theoretical Gain | Gain error (%) |
| --- | --- | --- | --- | --- | --- |

5. Lab Report Submission

The report should include:

a) Detailed Pre-lab works,

b) Calculations and discussion questions,

c) Data tables, conclusions and analysis, problems you find out during the lab work and the corresponding solutions, and any kind of reflection,

Group report written by not more than 2 people is acceptable as long as you all agree you will have the same mark.

You should submit your report before 13th April (for class 1) and 20th April (for class 2).