Circuit Analysis

Laboratory Exercise No. 2

Equipment learning and DAC test

Each bench position is equipped with a number of instruments, which you will be using throughout this course. The object of this session is to introduce you to two of these instruments and allow you to familiarize yourselves with their various controls and displays, and to study a 3-bit DAC.

1. RIGOL DG4202 Function Waveform Generator.

This instrument is used to generate a wide range of signals of different frequencies (time periods), amplitudes (sizes), and waveforms (shapes). The signals are applied to the circuit being investigated to establish how the design performs. An example of this might be to establish the range of frequencies that a Hi-Fi amplifier can reproduce.

The Power Soft-key is located at the left bottom and is used to turn the generator on or off. When the Power Soft-key is turned off, the generator is in standby mode and the generator is in power-off mode only when the power cable at the rear panel is pulled out. The Channel Output and Sync Output connecters are located at the bottom of the panel. Channel selection keys CH1 and CH2, their output switch keys Output1 and Output2 and the trigger keys Trigger1 and Trigger2 are above the connecters. The user interface displays the parameters and waveforms of the two channels at the same time. CH1 and CH2 can select the same or different waveforms from Sine, Square, Ramp, Pulse, Noise, Arbitrary waveform, Harmonic or User-defined Waveform. The keys for these waveforms are located at the top of right hand side of the panel beside a knob. The generator can output the first five waveforms. Other waveforms have to be input by edition. When the instrument is powered on, Sine waveforms are selected as current waveforms automatically. When a channel is selected, the corresponding channel backlight turns on. You can set the parameters by pressing the Menu Soft-keys beside the displays to active the corresponding menus to the left hand side. The knob located at the right top of the panel is used to increase (clockwise) or decrease(counterclockwise) the current highlighted number. And the Direction Keys under the knob are used to move the cursor to select the digit to be edited while using the knob to set the parameters. You can also use the Numeric Keyboards under the direction keys to input parameters.

We will now set the DG4000 to produce a 500Hz Sine wave with a peak-to-peak amplitude of 7.6 volts, offset of 0 DC, and start phase of 30. Use the DMM to measure the amplitude and frequency of this AC waveform. Note that the value it displays is the Root Mean Square (RMS) value of the wave, not the peak-to-peak measurement. It cannot give us any information about wave shape of the signal. To look at how a repetitive signal varies with time an oscilloscope is required.

Figure 1. Illustration of a sine wave

2. RIGOL MSO2202A Digital Oscilloscope.

This instrument is a sophisticated, state of the art, measuring system. You will see that the front panel consists of a rectangular Liquid Crystal Display (LCD) and large number of controls. The best method of learning what each of these controls does, is on a need to know basis.

The unit is switched on by pressing the (I)/(0) push button located on the bottom left hand side of the oscilloscope. The unit goes into a power-up test routine, and after a few seconds the opening screen is displayed. After a further few moments the measuring screen is displayed. So that everybody can start at the same point press the AUTO button which is located on the top right hand side of the front panel.

The display has a number of different areas. The large grid area is where the signal being examined is displayed. The right hand side of the display is used to show various options which are selected using the ten vertical pushbuttons next to the screen.

Fig. 2. Vertical Control                                Fig. 3. Horizontal Control                                  Fig. 4. Trigger Control

Vertical Control

In vertical control area shown in Figure 2, CH1 and CH2 are analog input channels. The two channels are marked by different colors which are also used to mark both the corresponding waveforms on the screen and the channel input connectors. Press CH1 and a new set of options will appear and press again to turn off the channel.

Press CH1 MENU and see the options. The top option is Coupling. This has three settings, DC, AC, and Ground, changed by pressing the top pushbutton. In the DC setting, the signal being measured is connected directly to the oscilloscope. When AC is chosen, an internal capacitor is placed in series with the input signal. This blocks any DC component that may be present. The Ground setting disconnects the measured signal from the oscilloscope input and connects the input to Earth.

The top knobs are Vertical Position controls which modify the vertical position of the current channel waveform. Turn clockwise to increase the position and turn counterclockwise to decrease. During the modification, the waveform would move up and down and the position message at the lower-left corner of the screen would change accordingly. Press down this knob to quickly reset the vertical position to zero. The bottom knobs are Vertical Scale controls which modify the vertical scale of the current channel. Turn clockwise to increase the scale and turn counterclockwise to decrease. During the modification, the display amplitude of the waveform would enlarge or reduce but the actual amplitude remains unchanged. The scale information at the lower-left corner of the screen would change accordingly. Press down this knob to quickly switch the vertical scale adjustment mode between ‘Coarse’ and ‘Fine’.

Horizontal Control

In this area shown in Figure 3, press MENU key to open the horizontal control menu under which to turn on or off the delayed sweep function, switch between different time base modes, switch between ‘Coarse’ and ‘Fine’ adjustment of scale as well as modify the horizontal reference setting. The Horizontal Scale is on the left hand side of Menu key which is used to modify the horizontal time base. Turn clockwise to reduce the time base and turn counterclockwise to increase the time base. During the modification, waveforms of all the channels will be displayed in expanded or compressed mode and the time message at the upper side of the screen would change accordingly. Press down this knob to quickly switch to the delayed sweep state. The Horizontal Position is on the right hand side of Menu key which is used to modify the horizontal position. The trigger point would move left or right relative to the center of the screen when you turn the knob. During the modification, waveforms of all the channels would move left or right and the trigger position message at the upper-right corner of the screen would change accordingly. Press down this knob to quickly reset the trigger position or the delayed sweep position.

The last option is Probe, used in conjunction with the slide switch on the body of the probe, to set the attenuation. It is normal to have both set to the same value. The 1OX setting is used when high frequency compensation is required to overcome the capacitive effect of the probe cable, or high voltages are to be measured.

We will now set up the oscilloscope to measure the signal produced by the MSO2202A function generator. The tip of the probe is connected to the Red crocodile clip on the generator output cable, and the Black probe crocodile clip is connected to the Black clip on the generator cable. Note that Black clip on the probe is connected to Mains Earth. Make sure the function generator is switched on and check that the controls are set up as before. Make sure the oscilloscope is switched on, and on this occasion the AUTO button has been pressed. Wait until a waveform is displayed.

If the display is not stable, go through the following procedure, otherwise skip it over.

1. Press CH1 MENU twice (any waveform trace should disappear).

2. Set Coupling to Ground.

3. Turn the Invert Off .

4. Turn the BW Limit Off.

5. Set the Probe to 1X.

6. Rotate the CH1 Position control to place the displayed horizontal line half way up the screen.

7. Rotate the TRIGGER LEVEL control to align the small arrowhead located on the right margin of the display with the same halfway point.

8. Press the TRIG MENU button.

9. Set the Type to Edge.

10. Set the Source to CH1.

11. Set the Slope to rising. (by pressing the Source/Slope button more options will come up)

12. Set the Coupling to AC. (go back to previous screen when set by selecting in on the right hand side of the screen)

13. Set the Mode to Auto.

14. Press CH1 MENU (waveform trace should now be present on the screen).

15. Set the Coupling to DC

A stable trace should now appear on the screen. To change the vertical range, rotate the CH1 VOLTS/DIV knob and set it to 1·00 V (shown at the bottom left hand corner of display).

Align the arrowhead at the top of the display to the halfway mark by turning horizontal set knob. This marker indicates the point in time that the trigger circuit fires.

Rotate the HORIZONTAL TIME/DIV knob to display at least one full cycle. The horizontal time/division setting is displayed in the middle of screen below the waveform. Set it to 500μS.

All measure results are being showed on the screen now. The frequency of the signal being displayed is shown at the bottom right hand corner of the display. How close is it to the frequency set up on the function generator? Adjust the FREQUENCY knob on the generator until the display reads 500Hz. Note the Amplitude value in Vpp, Vrms, period and frequency.

Next, turn back to DG4202 Function Waveform Generator, with the FUNCTION control, select a Square waveform and turn the Symmetry control on with the slide switch. Rotate the control and observe the changing waveform. Now observe the effect of applying a DC Offset to the signal. Note that if the dc offset is too large the oscilloscope will stop triggering unless the trigger is AC coupled. And you can repeat this operation to observe other waveforms.

You can press the HELP button if you like, to find out more from the built-in operating manual.

3. Digital to Analog (D/A) Converter

The digital-to-analog converter (DAC) transforms digital signals into analog form. A basic structure of a three-bit DAC is illustrated in Figure 2.

Figure 5 DAC

- Prelab

i) The position of the switch defines the digital ‘0’ or ‘1’ as the following:

ii) In Figure 5, S is open (RL is disconnected) and SD0, SD1 and SD2 are at ‘0’ position. Calculate the equivalent resistance Rab.

iii) When SD0, SD1 and SD2 are at different positions, calculate the corresponding open voltage Vaboc, and the equivalent resistance R0 seen from a-b terminals, respectively. Fill the results in Table 1 and determine the D/A relation(find k0, k1 and k2)：

Vaboc=k2D2+ k1D1+ k0D0

Table 1

| D2 D1 D0 | 000 | 001 | 011 | 010 | 011 | 100 | 101 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vaboc |  |  |  |  |  |  |  |  |
| Rab(R0) |  |  |  |  |  |  |  |  |

- lab section

i) Measurement of Rin seen from the voltage source

Disconnect the load and the source. Measure the input resistance seen from the voltage source when SD0, SD1 and SD2 are at different positions.

Table 2

| D2 D1 D0 | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rin |  |  |  |  |  |  |  |  |

ii) Measurement of Vaboc and R0 seen from a-b terminals

Connect the voltage source and set the voltage Vs to 8V with 1A current limit. When SD0, SD1 and SD2 are at different positions, measure the corresponding open voltage Vaboc, and the equivalent resistance R0 seen from a-b terminals, respectively. Fill the results in Table 3 and compare the results to the values in Table 1.

Table 3

| D2 D1 D0 | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vaboc |  |  |  |  |  |  |  |  |
| Rab(R0) |  |  |  |  |  |  |  |  |

iii) Measurement of Vab when load RL is connected

Connect the voltage source and set the voltage Vs to 8V with 1A current limit. Connect the load 2k. When SD0, SD1 and SD2 are at different positions, measure the voltage Vab. Change the load by a 1k resistor and repeat the measurement of Vab. Fill the results in Table 4. What conclusion can you get?

Table 4

| D2 D1 D0 | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vab(RL=2k) |  |  |  |  |  |  |  |  |
| Vab(RL=1k) |  |  |  |  |  |  |  |  |

4. Lab report submission

This practical can be carried out in pairs. The report should be completed individually. The report should include:

a) typical pictures and records of instrument learning,

b) picture of the circuit you build,

c) the testing results,

d) the problems you find out during the lab work and the corresponding solutions, and any kind of reflection.