1. (5分)  Answer the following questions:

(1) Please write down three types of power diodes.

(2) Which type are Thyristor and IGBT, current-driven device or voltage-driven device?

(3) Why the MOSFET can be easily paralleled to conduct higher current .

(1) 功率二极管分哪三种类型？(2分)

(2) 分别说出晶闸管和IGBT属于电压控制还是电流控制型器件？(2分)

(3) 为什么MOSFET器件容易实现并联均流？(1分)

答案：

(1) 功率二极管分为普通整流二极管、快恢复二极管、肖特基二极管。

(2) 晶闸管属于电流控制型器件，IGBT属于电压控制型器件。

(3) MOSFET能够并联使用提高通流能力，是因为MOSFET的通态电阻具有正的温度系数，即通态电阻随温度升高为增大，因此具有均流的作用(电流大→温度升高→电阻增大→电流减小)。

2. (4分)  The switching waveforms of a power electronics device are shown in Fig.1. If Vo=50V, Io=10A, on-state resistance is 20mΩ. The turn-on delay and turn off delay time are neglected. Answer the following questions:

某一电力电子器件的开关波形如图1所示。如果Vo=50V, Io=10A，通态电阻为20mΩ。忽略开通延迟和关断延迟时间。回答以下问题。

(1) What is the switching power loss of the switch? 开关管的开关损耗是多少？(2分)

(2) What is the total power loss of the switch? 开关管的总损耗是多少？(2分)

Fig. 1  Switching waveforms of a power electronics device

答案：

(1)

Ps=4.75W

(2)   Pon=0.782W

P=Ps+Pon=5.532W

3. (8分)  In a Buck converter, consider all components to be ideal. Let input voltage Vd be 50~80V, output voltage Vo = 40V, and L=60μH, fs = 40kHz.  在Buck变换器中，考虑所有元件都是理想的。设输入电压Vd为50~80V，输出电压Vo为40V，L=60μH，开关频率为40kHz。

(1) Draw the Buck converter circuit diagram. 画出Buck变换器电路图。(1分)

(2) Draw the waveforms of  vL、iL、iC. 画出vL、iL、iC的波形。(2分)

(3) Calculate the minimum output power Pomin that will keep the converter operating in a continuous-conduction mode for all range of Vd. 计算全输入电压范围内能够使变换器工作在CCM模式下的最小输出功率Pomin。(3分)

(4) What will happen if keep D=0.5 and the load R is open? 如果占空比固定而负载突然开路，会出现什么结果？(1分)

(5) Which is big for the Switch utilization in condition of D=0.2 and D=0.8？比较在D=0.2和D=0.8两种情况下哪个开关利用率高？(1分)

答案：

(1) Buck变换器电路图：

(2) vL、iL、iC的波形：

(3)计算最小输出功率Pomin：

Vo =DVd，Vd =50~80V，Vo =40V，D=0.5~0.8，

Vo =40V，L =60uH，Ts =25us，取最小值D =0.5，

代入得IoB=4.16A，

Pomin =Vo *IoB = 166.6W

(4)如果占空比固定而负载突然开路，输出电压会升高，直至输出电压等于输入电压；

(5) D=0.8时开关利用率高。

4. (8分)  In a Buck-Boost converter, consider all components to be ideal. Let input voltage Vd = 24V, D=0.6, and L=100μH, fs = 50kHz, Po=360W. 在Buck-Boost变换器中，假设所有元件都是理想的。设输入电压Vd= 24V，占空比D=0.6，电感为100μH，fs = 50kHz，Po=360W。

(1) Draw the Buck-Boost converter circuit diagram. 画出Buck-Boost变换器电路图。(1分)

(2) Derive the relationship between the output voltage Vo and input voltage Vd, when the inductor current is continuous. 推导电感电流连续时输出电压Vo与输入电压Vd之间的关系式。(2分)

(3) If the output voltage ripple ratio ∆Vo/Vo is less than 2%，please calculate the minimum value of C in this converter. 如果输出电压纹波比∆Vo/Vo小于2%，请计算该变换器中电容C的最小值。(2分)

(4) What is the voltage stress of the switch? 开关管的电压应力是多少? (1分)

(5) What is the advantage and disadvantage of this converter? 该变换器的优缺点是什么？(2分)

答案：

(1) Buck-Boost变换器电路图：

(2) 输出电压Vo与输入电压Vd的关系式根据伏秒平衡推导：

(3)

Vo=36V， ∆Vo/Vo<2%， ∆Vo=0.72V

Po =360W， Vo =36V ，Io =10A，Ts =20us，D =0.6，

代入得 Cmin = 166.6 μF

(4)开关管的电压应力： VT=Vd+Vo=24+36=60V

(5) 优点：电路可升压可降压。缺点：输入和输出反极性，开关管的电压、电流应力较大。